DIGITAL CLOCK AND ALARM
PROJECT REPORT

Department of Computer & Software Engineering

College of E&ME, NUST, Rawalpindi

Course: EC-210 Logic & Sequential Circuit Design

Course Instructor: Dr Ahsan Shazad

Lab Instructor: LE Usama Shaukat

Section: CE-47-B

Group Members:

1. Lameea Mubashir Khan — 570918

2. Tooba Fatima — 577687

3. Alizay Hassan — 551640

Submission Date: May 2026

Table of Contents

1. Abstract

2. Objectives

3. Introduction

4. Problem Statement

5. Design Approach

6. Software Name and Usage

7. Tasks

8. Simulation and Testing

9. Challenges Faced

10. ICs Used Summary

11. Conclusion

12. References

Abstract

This project presents the design and implementation of an advanced digital clock system using discrete logic ICs. The architecture uses CD4026 decade counters integrated with seven-segment displays to provide accurate timekeeping. The project includes alarm functionality, time adjustment, and time-zone adjustment without the use of microcontrollers.

Objectives

• Display hours, minutes, and seconds using seven-segment displays.

• Ensure accurate rollover at 59 seconds, 59 minutes, and 23 hours.

• Implement manual time adjustment using push buttons.

• Design a digital alarm system using comparator logic.

Introduction

Digital clocks are commonly implemented using microcontrollers; however, this project demonstrates the implementation using only combinational and sequential logic circuits. The project emphasizes the understanding of digital electronics, counter circuits, logic gates, and timing mechanisms.

Problem Statement

The goal of this project is to design a fully functional digital clock capable of displaying time in HH:MM:SS format, allowing manual adjustment and alarm functionality using logic ICs only.

Design Approach

Hardware Components

• CD4026 Decade Counter and 7-Segment Decoder

• 555 Timer IC

• 74LS86 XOR Gates

• 74LS08 AND Gates

• 74LS04 Inverters

• 74153 Multiplexers

• BC547BP Transistor

• 7-Segment Displays

• Push Buttons and Switches

• Buzzer

Clock Pulse Generation

A 555 timer IC generates a stable 1Hz pulse which drives the seconds counter. Carry-out connections between counters increment minutes and hours automatically.

Software Name and Usage

Software Used: Proteus Design Suite

Proteus Design Suite was used to design, simulate, and test the complete digital clock system. The software allowed verification of timing, counter rollover, and alarm functionality.

Tasks

Task 1: Basic Clock Functionality

Implemented seconds, minutes, and hours counters using CD4026 ICs connected to seven-segment displays.

[Insert Proteus Screenshot Here]

Task 2: Time Setting Functionality

Added push-button switches to manually increment hours and minutes for setting the current time.

[Insert Proteus Screenshot Here]

Task 3: Alarm Functionality

Designed comparator logic using XOR gates to activate a buzzer when the current time matches the alarm time.

[Insert Proteus Screenshot Here]

Task 4: Time Zone Adjustment

Implemented logic for changing the displayed time zone using multiplexers and additional pulse control.

[Insert Proteus Screenshot Here]

Simulation and Testing

• Verified correct rollover from 59 to 00 for seconds and minutes.

• Tested alarm activation at the programmed time.

• Confirmed stable 1Hz pulse generation using the 555 timer.

Challenges Faced

• Synchronization between counters without glitches.

• Designing reset logic for 24-hour rollover.

• Ensuring instant buzzer activation for the alarm.

ICs Used Summary

| IC Name | Function | Quantity |
| --- | --- | --- |
| 4026 | Decade Counter / 7-Segment Decoder | 10 |
| 555 Timer | Pulse Generator | 1 |
| 74LS86 | XOR Gates | 10 |
| 74LS08 | AND Gates | 4 |
| 74LS04 | Inverters | 3 |
| 74153 | Multiplexer | 4 |

Conclusion

The project successfully implemented a fully functional digital clock using logic ICs. The system demonstrated accurate timekeeping, alarm functionality, and manual time adjustment while reinforcing concepts of digital logic design.

References

• Proteus Design Suite Documentation

• CD4026 Datasheet

• 555 Timer IC Datasheet

• Digital Logic Design Course Notes